Assessment to be completed at project initiation to inform project scope.

PROJECT INFO & COMPLETE STREETS ASSESSMENT -  STREAMLINED

For projects up to $100,000 and spot improvement projects

Project Name:  21st Ave S – College to Walker Walkway

Project Location:  21st Ave S – S College St to S Walker St

Program(s) / Funding Source(s):  PMP New Sidewalks

Project Manager / Owner:  David Burgesser

Outreach Lead:  David Burgesser

TOD lead: John Marek

Project Budget/Estimate:  $200,000

Billing Number(s):  TBD

DotMaps ID:  76310

Initial Outreach Completed: Yes, contacted neighbors requesting improvements in this area.

Desired Delivery Date / Time Constraints: Q3, 2024

Move Seattle Accomplishments: 1 blocks of new sidewalks (1x M-SWIN)

Project Delivery PM: (To be filled by PDD)

Project Delivery Designer: (To be filled by PDD)

Delivery Method(s): (To be filled by PDD)

WO issue deadline: (To be filled by PDD)

Description of Scope:

- New 6-8’ asphalt walkway with pre-cast concrete wheel stops on the west side of 21st Ave S between S College St and S Walker St. Include a planting strip/conveyance swale with trees between the walkway and the roadway wherever grades allow or raised asphalt walkway supported by wheel stops (if feasible).

- Construct missing curb ramps/at-grade flush transitions on the SW and NW corners of 21st Ave S & S Walker St.

- Mark crosswalks on the north and west legs of 21st Ave S & S Walker St connecting these ramps to facilitate the final connection to the Giddens School/Lake Washington Girls Middle School.

- Construct curb ramps/at-grade flush transitions on all four corners of 21st Ave S & S College St. The east-west facing ramps on the SW and SE corners are under design by SSRP (Tony Chang) and planned for 2024 construction (designs to be provided).

- Add 20MPH school zone signage on S College St between 20th Ave S & 22nd Ave S (on crosswalk approaches).

- Install painted curb bulbs on the NW and SW corners of the 21st Ave S & S College St intersection to narrow the intersection throat to 18’.

- Install a marked crosswalk on the west leg of 21st Ave S & S College St.

  - Tier 1 crossing connecting directly into the signed school zone on the north side of the intersection.

- Install speed cushions on the westbound approach to the 21st Ave S & S College St intersection.

  - 85% speed: 32.7MPH (collected e/o 18th Ave S in 10/2022)

  - AWDT: 5,806 (collected e/o 18th Ave S in 10/2022)

  - Slope (21st to 22nd): 3%

Project Log:

- 7/10/2023, David Burgesser - Conducted site visit to verify extents of school zone and observe grade challenges and site conditions.

- 7/14/2023, David Burgesser – Reached out to SSRP team (Tony Chang) to verify curb ramp scope at 21st Ave S & S College St. Tony provided design plans and notified that the curb ramp construction would not occur until 2024 at the earliest.

Reviewed & Approved:

- John Marek (Date/Initials – Traffic Ops)

- David Burgesser (Date/Initials – Program Owner)

- Reiner Blanco (Date/Initials – Project Delivery)

- Tom Hewitt (Date/Initials – ADA Coordinator)

Concept Design

21st Ave S – College to Walker